Procedure: 1x Screw Cell

Order Description

| Client |  |  |
| --- | --- | --- |
| Order # |  |  |
| Order component description | Screw Cell 1x |  |
| Version | 1.3RC HN 5mm rev4 |  |
| Number of items / Start date |  |  |
| Batch # |  |  |

Procedural Checklist  :

| # | Procedure | Check / initial |
| --- | --- | --- |
| 1 | 3D print: Form 3, ___________ resin |  |
| 2 | IPA wash prints __ min #1, __ min #2 |  |
| 3 | Eliminate resin residues: repeat 3a and 3b as many times as required to remove resin residues (reflective sticky spots) and clear channels |  |
| 3a | Power wash external surfaces and flush cell channels >2s with IPA (WaterPik) |  |
| 3b | IPA wet, vacuum and/or air gun and/or wipe excess IPA + resin residues; avoid surface damage, especially to chip interface region |  |
| 4 | >30 min air dry to vacate surface-absorbed resin solvent |  |
| 5 | UV post-cure __ min @ __°C |  |
| 6 | Remove supports and protruding support marks to QC spec - Sanding and filing external surfaces - Removal of electrolyte well support in front of a channel (pipette area) - Removal and filing (if required) of electrolyte well external support (cap insertion area) - Removal and filing (if required) of screw and nut hole supports |  |
| 7 | Vacuum/brush/blast off coarse dust |  |
| 8 | Clean in small cell batches via sonication for 5min in 50%v/v IPA/DI to dilute and clear out small debris; Typical recipe: 400mL fluid for 5 - 6 cells, max 2 batches in same solution. Chip well regions should not be touching adjacent parts during the process. |  |
| 8a | (Optional) Apply power wash with IPA to remove stubborn sanding residues if present |  |
| 9 | Air gun cell and channels to vacate cleaning fluid |  |
| 10 | Perform final QC and assemble cell (top, bottom, M2 screw x2, M2 nut x2) |  |

Notes:

Production operators / Initials